Fundamental of Computer Science

Homework Set 4

October 29, 2025

1. (2’) Suppose a multiprogramming operating system does not terminate a process performing I/O operation. Each time slice is 50 ms. It normally takes 12 ms to position the disk's read/write head over the desired track and another 8 ms to rotate for the desired data.

- How much of a time slice can be spent waiting for a read operation (from a disk to take place)?

- If the machine is capable of executing 10 instructions each microsecond (μs), how many instructions can be executed during this waiting period?

For a), fill a percentage in the form (e.g., 20%). For b), fill an integer in the form (e.g., 100).

|  | a) | b) |
| --- | --- | --- |
| Answer | 40% | 200000 |

2. (1’) A process is said to be I/O-bound if it requires a lot of I/O operations, whereas a process that consists of mostly computations within the CPU/memory system is compute-bound. If a compute-bound process and an I/O-bound process are both waiting for a time slice, which should be given priority? (OS could terminate a process performing I/O operation and allow another process to run while the first is waiting.)

Fill an uppercase letter in the form.

- I/O-bound process   B. compute-bound process

| Which should be given priority? | B |
| --- | --- |

3. (3’) Suppose each time slice in a multiprogramming system is 50 ms and each context switch requires 1 μs.

- What fraction of the machine's time is spent actually performing compute-bound processes?

- What would this fraction be if each process executes an I/O request a microsecond (μs) after its time slice begins? (Assume OS terminates a process performing I/O operation and allows another process to run while the first is waiting, and there are enough processes to keep the CPU busy.)

- Suppose there are 5 compute-bound processes running concurrently. What is the average waiting time for each process before it gets the CPU again(in ms, ignoring context switch time)?

Fill your answers in the following form (fractions like 100/101, use ‘/’ to separate two integers without spaces).

|  | a) | b) | c) |
| --- | --- | --- | --- |
| Answer | 50000/50001 | 1/2 | 200ms |

4. (4’) Consider the states of a process. Determine whether the following description is correct and fill ‘True’ or ‘False’ in the forms.

4.1 After a process is created, it immediately enters the running state.

4.2 When a process is waiting for an event (such as I/O completion), it transitions from the running state to the waiting state.

4.3 Once a process is woken up, it immediately resumes execution.

4.4 In a single-core CPU, at most one process can be in the running state at any given time.

|  | 4.1 | 4.2 | 4.3 | 4.4 |
| --- | --- | --- | --- | --- |
| Answer | False | True | False | True |